Network Registration:

AT+CREG

AT+CGREG

Settings Bearer IP Application

AT+SAPBR

AT+HTTPINIT

AT+HTTPPARA

AT+HTTPDATA

AT+HTTPREAD

AT+HTTPACTION

AT+HTTPTERM

Select SMS Message Format

AT+CMGF

Send SMS

AT+CMGS

Battery: AT+CBC
Charging, Percent, MilliVolts

# Network:

Network Registration Status:

AT+CREG?				(AT+CGREG (2G oderso))

+CREG 0,1-> (Sucht nicht), (Home Network)

Bearer Info

AT+SAPBR=2,1			(2 get info, 1. Bearer)

+SAPBR: 1,3,"0.0.0.0"		(1. Bearer, 3 Closed)

AT+SAPBR=1,1			(1 öffnen, 1. Bearer)

+SAPBR: 1,1,"10.42.71.191"	(1. Bearer, 1 Offen, ip)

AT+SAPBR=4,1			(2 get AP info, 1. Bearer)

+SAPBR:

CONTYPE: GPRS

APN:

PHONENUM:

USER:

PWD:

RATE: 2

HTTP

AT+HTTPINIT

AT+HTTPPARA="URL","http://google.com"			(Setzt URL)

AT+HTTPPARA="CID",1					(Setzt 1. Bearer)

AT+HTTPACTION=0						(0 – HTTP Get)

+HTTPACTION: 0,301,219					(301 moved, 219 – Data Len)

+HTTPACTION: 0,200,18618				(200 OK, 18618 – Data Len

AT+HTTPREAD

<!Doctype HTML> etc…. </html>

AT+HTTPTERM						(Beendet HTTP Mode)

AT+SAPBR=0,1						(0 Close, 1. Bearer)

# POST DATA

Wie in Network:

AT+HTTPDATA=<DataLen>,<WaitForInputTimeMs>

AT+HTTPDATA=10,10000
DOWNLOAD				(Von Sim geschickt)

TEST					(Post Data)

AT+HTTPACTION=1			(1-Post)
+HTTPACTION: 1,603,0		(1-Post, 603 Fehlercode DNS (HTTPS geht nicht), 0-Length)
+HTTPACTION: 1,200,360		(1, 200 OK, 360 – Length Data)

AT+HTTPREAD

{

"args": {},

"data": "TEST\nTEST\n",

"files": {},

"form": {},

"headers": {

"Accept": "*/*",

"Content-Length": "10",

"Host": "httpbin.org",

"User-Agent": "SIMCOM_MODULE",

"X-Amzn-Trace-Id": "Root=1-67b5fc86-2c27111747bdae5175fc23dd"

},

"json": null,

"origin": "80.187.87.72",

"url": "http://httpbin.org/post"

}

OK

AT+HTTPTERM

# SMS:

AT+CMGF=?		(Get List of Supported Formats)
+CMGF: (0,1)		(0 pdu, 1 text)

AT+CMGF?		(Get active Text Mode)
+CMGF: 0		(0 PDU Mode) (Binary Bit Stream vllt interessant)

AT+CMGF=1		(Set Text Mode)

AT+CMGS="+4915739119874"	(Receiver Number)

Message

CTRL+Z		(Send with Code: 0x1a)

+CMGS: 6		(6 <da> ? Kein Ahnung, fortlaufende Nummer irgendwie)